- CLEAN DEVELOPMENT MECHANISM

- PROJECT DESIGN DOCUMENT FORM (CDM-SSC-PDD)
Version 03 - in effect as of: 22 December 2006

# CONTENTS

A. 	General description of the small scale project activity

B. 	Application of a baseline and monitoring methodology

C. 	Duration of the project activity / crediting period

D. 	Environmental impacts

E. 	Stakeholders’ comments

## Annexes

Annex 1:  Contact information on participants in the proposed small scale project activity

Annex 2:  Information regarding public funding

Annex 3:  Baseline information

Annex 4:  Monitoring Information

Revision history of this document

| Version Number | Date | Description and reason of revision |
| --- | --- | --- |
| 01 | 21 January 2003 | Initial adoption |
| 02 | 8 July 2005 | - The Board agreed to revise the CDM SSC PDD to reflect guidance and clarifications provided by the Board since version 01 of this document. - As a consequence, the guidelines for completing CDM SSC PDD have been revised accordingly to version 2. The latest version can be found at <http://cdm.unfccc.int/Reference/Documents>. |
| 03 | 22 December 2006 | - The Board agreed to revise the CDM project design document for small-scale activities (CDM-SSC-PDD), taking into account CDM-PDD and CDM-NM. |

SECTION A. 	General description of small-scale project activity

A.1 	Title of the small-scale project activity:

>>

Shree Chhatrapati Shahu RE Project

Version 5

12/11/2007

A.2.	Description of the small-scale project activity:

>>

The project activity is undertaken by Shree Chhatrapati Shahu Co-operative Sugar Factory Ltd., Kagal, Kolhapur, India.  The project seeks to generate renewable electricity from bagasse, a renewable biomass material, that is produced from the milling of the sugar cane.  The electricity generated will be exported to the state electricity grid which in the absence of project activity would be generated primarily through the combustion of fossil fuels leading to emissions of greenhouse gas.  The factory will also make significant contribution to the sustainable development of the area surrounding the plant.

The project activity involves the installation of a new boiler and turbine generator at the sugar factory.  This will enable the export of 7.43MW of electricity to the grid in the season and 10.69MW in the off-season.

| New |  | Boiler | Turbine |
| --- | --- | --- | --- |
| Capacity | 70TPH |  | 12.5MW |
| Operating Temperature | 485oC |  | 480oC |
| Operating pressure | 67kg/cm2 |  | 64kg/cm2 |
| Manufacturer | KCP |  | Siemens |
| Type | Water Tube |  | Condensing cum extraction |

The existing power demand at the factory is satisfied by the following boilers and turbines.

| Existing |  | Boiler | Turbine |
| --- | --- | --- | --- |
| Capacity |  | 2 X 27TPH ,40TPH and 20TPH | 1.5MW and 2.5MW |
| Operating Temperature |  | 340oC | 310-320oC |
| Operating pressure |  | 21kg/cm2 | 18-20kg/cm2 |
| Manufacturer |  | Backan/Wolf and KCP | BHEL & Triveni |
| Type |  | Water Tube | Back pressure |

In the existing system, two boilers with capacities of 27TPH and one with a 20TPH capacity will be decommissioned.  The 40TPH boiler will produce steam for the mills of sugar plant.  All the existing turbines will be decommissioned after stabilization of the new power plant.

Contribution to Sustainable development

The factory makes a significant contribution to sustainable development not just indirectly through its co-operative structure but also directly.  The co-operative structure of the sugar factory results in all the profits of the factory being returned to the members (who are all farmers supplying cane to the factory) in the form of a higher cane price.  Therefore any improvements carried out at the factory will filter back to the farmers.  The factory is also directly involved in a number of extension activities with the farmers and provides educational and health facilities to its workers.  The project itself is expected to increase employment opportunities for the local population.

The extension work carried out by the factory is wide ranging – from the guaranteeing of loans provided to farmers by banks through to the provision of seed and fertilizer at nominal rates.  The factory serves about 25,000 farmers from 91 villages located in the surrounding areas.

The project will contribute to the sustainability of the factory and thus foster further economic development in the surrounding area through the strengthening of agricultural activities.  The generation of renewable electricity will also reduce the dependence on existing and planned grid based fossil fuel based generation.  This will have a positive impact not only through the reduction in emissions of greenhouse gases associated with such generation, which is predominantly coal based (see section on determination of the baseline), but also through a reduction in the emissions of other harmful gases (NOx and SOx) that arise from the combustion of coal.

The proposed CDM project is undertaken in the co-operative sugar sector in Maharashtra and will act as a catalyst to others wishing to install grid based generation.  This is of significant importance in the state given that there are over 150 co-operative factories that contribute to the livelihoods of the rural population.

A.3. 	Project participants:

>>

| Name of Party involved ((host) indicates a host Party) | Private and/or public entity(ies) project participants (as applicable) | If Party wishes to be considered as a project participant |
| --- | --- | --- |
| India (host) | Cooperative entity: Shree Chhatrapati Shahu Co-operative Sugar Factory Ltd | No |
| UK | Private entity: Agrinergy Ltd | No |

The official contact for the project activity will be Shree Chhatrapati Shahu Co-operative Sugar Factory Ltd as listed in Annex I.

A.4. 	Technical description of the small-scale project activity:

A.4.1.  Location of the small-scale project activity:

>>

A.4.1.1.		Host Party(ies):

>>

India

A.4.1.2.		Region/State/Province etc.:

>>

Maharashtra state

A.4.1.3.		City/Town/Community etc:

>>

Kolhapur district, Kagal town

A.4.1.4.		Details of physical location, including information allowing the unique identification of this small-scale  project activity :

>>

The complete postal address of the sugar factory is:

Shree Chhatrapati Shahu Co-operative Sugar Factory Ltd

Shrimant Jayashingrao Gatge Bhavan

Kagal – 416 216

District Kolhapur

Maharashtra, India

The geographical location of Kolhapur is Latitude: 16.42oN and Longitude: 74.16oE.  The sugar factory is located at a distance of 20km from Kolhapur.

A.4.2.  Type and category(ies) and technology/measure of the small-scale  project activity:

>>

Type I – Renewable Energy Projects

I D – Grid connected renewable electricity generation

All the required guidelines will be met for compliance with local safety and environment legislation.  Consents from state pollution board in the past demonstrate that the project proponents have followed these guidelines and all future consents will be based on any new guidelines specified by the pollution control board.  The technology is available in India and the technical support/training will be provided at commissioning.

A.4.3	Estimated amount of emission reductions over the chosen crediting period:

>>

A 7 year renewable crediting period has been chosen.

| Year | Estimation of annual emission reductions in tonnes of CO2e |
| --- | --- |
| Year 2007-2008 | 39,202 |
| Year 2008-2009 | 39,202 |
| Year 2009-2010 | 39,202 |
| Year 2010-2011 | 39,202 |
| Year 2011-2012 | 39,202 |
| Year 2012-2013 | 39,202 |
| Year 2013-2014 | 39,202 |
| Total estimated reductions (tonnes of CO2e) | 274,414 |
| Total number of crediting years | 7 |
| Annual average of the estimated reductions over the crediting period (tCO2) | 39,202 |

A.4.4.  Public funding of the small-scale project activity:

>>

No public funds will be invested in the project activity.

A.4.5.  Confirmation that the small-scale project activity is not a debundled component of a large scale project activity:

Appendix C, paragraph 2 of the Simplified Modalities and Procedures for Small-Scale CDM project activities states:

“A proposed small-scale project activity shall be deemed to be a debundled component of a large project activity if there is a registered small-scale CDM project activity or an application to register another small-scale CDM project activity:

•		With the same project participants;

•	In the same project category and technology/measure; and

•	Registered within the previous 2 years; and

•	Whose project boundary is within 1 km of the project boundary of the proposed small-scale activity at the closest point.”

As there is currently no registered CDM project at the site either large scale or small scale, the project will meet the criteria on debundling.

SECTION B. 	Application of a baseline and monitoring methodology

B.1.	Title and reference of the approved baseline and monitoring methodology applied to the small-scale project activity:

>>

I D – Grid connected renewable electricity generation

Version 10, 23 December 2006

B.2	Justification of the choice of the project category:

>>

Grid connected renewable electricity generation

The project activity will establish a grid connection and the electricity supplied from the project activity, through the combustion of bagasse (a renewable biomass material), would be expected to supplement existing and planned electricity generation from the grid, the majority of which is fossil fuel based.  The project activity therefore satisfies the applicability condition relating to renewable biomass and supply of electricity to a distribution system that is currently operating on fossil fuel.

In the project activity, the new turbine generator has a capacity of 12.5MW, which therefore qualifies under the small scale rules as this is below the eligibility limit of 15MW.  No emission reductions are being claimed for heat generation from the new boiler, therefore we do not need to account for the thermal capacity of the boiler.

The new renewable electricity generating unit is physically distinct and the existing units will not be operated once the new unit starts generating.

B.3.	Description of the project boundary:

>>

In line with the guidance in “Appendix B of the simplified modalities and procedures for small-scale CDM project activities” the boundary for category I.D. projects “encompasses the physical, geographical site of the renewable generation source”.  The project boundary includes the equipment installed for the operation of the power plant, the main elements of which are the boiler, turbine generator, condenser, water treatment plant, effluent treatment plant, electrostatic precipitator, step up plant/transformers, transmission lines and the Western grid.  The emissions from the transport of outside biomass have been included as project emissions and are therefore included in the boundary.

TG 12.5MWW

BOILER

70tph

67 kg/cm2

CDM Boundary

To plant

STEAM HEADER

To grid

Outside biomass

For the purposes of the project activity the relevant grid is defined by the power generating units serving the same grid as the project activity.  In the case of India there are regional grids which facilitate the transfer of electricity between states and which are supplied by central sector power stations operating in the region.  Maharashtra is part of the Western Region (along with Gujarat, Madhya Pradesh, Chhattisgarh and Goa) and we have therefore undertaken an analysis of the Western grid to determine the carbon emission factor.  This provides a complete analysis of the power plants that the project will affect.  We do not believe that the national grid is appropriate given the limited interconnectivity of the regional grids and the size of the project relative to national power generation capacity.

B.4.	Description of baseline and its development:

>>

Referring to Appendix B of the Simplified Baseline and Monitoring Methodologies the baseline for the project activity is the MWh exported to the gird multiplied by the grid emission coefficient (tCO2/MWh) calculated using approach 9 (a) - A combined margin (CM), consisting of the combination of operating margin (OM) and build margin (BM) according to the procedures prescribed in the approved methodology ACM0002.  We do not include power to the factory as our baseline assumes that sugar factories are power independent.  In the specific case of the project activity, the baseline scenario is that the sugar factory remains self sufficient in power.  That this is the case and that the project activity is not the baseline has been demonstrated in section B.5

Data used to determine the baseline scenario

| Baseline data | Key information | Source |
| --- | --- | --- |
| Grid generation | Generation data of grid based generating units | Central Electricity Authority |
| Grid emissions | Fossil fuel consumption of grid based generating units | Central Electricity Authority |
| Capacity expansions | Timing of expansions to determine build margin | State electricity boards and generating companies |
| Net calorific value of fossil fuel used in grid plants |  | India’s National Communication to UNFCCC or IPCC |
| Emissions factor of fossil fuel used in grid plants |  | India’s National Communication to UNFCCC or IPCC |
| Oxidation factor of fossil fuel used in grid plants |  | IPCC |

The baseline emission reductions would be from the electricity exported by the project activity to the grid and the same has been detailed in section B.6.1.

B.5.	Description of how the anthropogenic emissions of GHG by sources are reduced below those that would have occurred in the absence of the registered small-scale CDM project activity:

In line with attachment A to appendix B of the simplified M&P for small-scale CDM project activities demonstration of additionality focuses on the barriers facing the project – specifically the investment and technological barrier.  In showing that the project is additional we demonstrate that it is not part of the baseline scenario, which in the case of Shree Chhatrapati Shahu RE Project is that the grid continues to operate and expand based on predominantly fossil fuel generation.

The main barrier to undertaking bagasse based grid supply using a high-pressure configuration (which is more capital intensive than a low pressure configuration) in cooperative sugar mills is access to finance. Cooperative sugar mills distribute their entire profits through the cane price to their members (the farmers).  This limits access to equity and severely reduces the financial strength of cooperative mills and their ability to obtain the finance required to undertake investments.

This situation is compounded by the fact that members typically prefer short term returns over the longer term returns which investments in bagasse based grid supply entail.

In 2003/4 121 out of 137 of the sugar factories in Maharashtra were cooperatives – Maharashtra is the largest sugar producing state in India, but because of the cooperative structure, uptake of bagasse cogeneration for export to the grid has been minimal.  There is a large volume of evidence illustrating the barriers facing uptake of electricity generation for export to the grid in the cooperative sugar sector including:

http://sify.com/finance/fullstory.php?id=14180638

http://www.teriin.org/events/docs/pdf/6wagh.pdf

http://repositories.cdlib.org/cgi/viewcontent.cgi?article=1019&context=ucias

http://www.wbcsd.org/web/projects/climate/ghg-forum2006/kala.pdf

As highlighted in the 2005 University of California study referred to above:

“…. it is evident that the primary barrier to bagasse cogeneration in cooperative sugar mills is their financial weakness. Sugar cooperatives have been an excessive burden on the state exchequer and have defaulted on loans running into millions of Rupees.  For this reason, financial institutions run by the national government and other private financial institutions have declined any further lending to the cooperatives. Although our field research shows awareness of the practice of bagasse cogeneration, the financial health of the cooperatives has prevented them from seeking the necessary investment for such projects. Thus the institutional practices of the cooperatives and their poor financial health have prevented them from making the investment in technology upgrade that some of their private counterparts have been able to make.”

The Shree Chhatrapatti Shahu project was undertaken with the assistance of a project financed by the UK Foreign and Commonwealth Office’s (FCO) Global Opportunities Fund (GOF).  The GOF is the FCO’s newest programme budget and has been created to fund projects around the world relating to the FCO’s eight strategic international policy priorities.

Electricity generation from sugar-cane residues combats climate change, contributes to energy security and helps sugar mills diversify their revenue streams.  Around the world many entities involved in the milling of sugar cane are benefiting from this technology.  However, this success has not been replicated in the vast Indian cooperative sugar sector, largely due to barriers to investment.  The GOF funded programme evaluated novel funding structures, including carbon finance, which can overcome these barriers.  It was expected that the study would result in an actual investment at a sugar mill.

The Shree Chhatrapatti Shahu project has successfully overcome the above barriers, and the role of carbon finance in achieving this is very important.  Firstly, the project applied to, and was accepted by, the Austrian JI/CDM programme and an ERPA has been signed.  The project is financed by a loan from the Bank of India and through a loan from the Sugar Development Fund. The CER revenue will be in Euros with a first rate counterparty and the CER sales price is fixed.  This has provided assurance to the bank in lending to a cooperative and has also helped the management of the factory convince stakeholders (farmers) of its viability.

In addition to the fact that without the CDM and a CER sale agreement with a first rate counterparty the project would not have been able to reach financial closure, the return of the project without the benefit of the CDM is below that expected in private sugar factories, and that required to compensate for the risks inherent in such an investment. Nearly 40 per cent of Indian cooperative sugar mills are losing money and facing closure.  In the state of Maharashtra 71 mills are sick, and in Vidarbha alone (where Shree Chhatrapatti Shahu is located), 17 are either sick or have already gone under liquidation. The financial situation of the Maharashtra State Electricity Board is not strong, and the payment risk on the Power Purchase Agreement is real. A reasonable investment horizon given these risks is 10 years, with a terminal value for the scrap value of the equipment at year 11.  A conservative investment benchmark is 20%.  The IRR without CDM revenue is well below this level.

The project activity also faces technological barriers.  The alternative to the project activity is to continue to operate a low pressure cogeneration system.  The project activity will use a high pressure boiler and this presents a new set of operational challenges for the management - low pressure configurations are common in Maharashtra and are technically easier to operate.  Furthermore, technological and operational barriers face the project relating to supply of electricity to the grid.  Such barriers, including synchronization, variations in the grid voltage and frequency and grid failure, affect all power plants.  However, the relative impact of these on the project activity is higher given that the primary activity of the factory and management is the processing of members’ sugar cane and not production of electricity.

B.6.	 Emission reductions:

B.6.1. Explanation of methodological choices:

>>

Emission Reductions

The emission reductions from the project activity are calculated by the application of the following equation:

ERy=ERelectricity,y−PEy−Ly

Where:

ERy	emission reductions of the project activity during the year y in tons of CO2

ERelectricity, y	emission reductions due to the displacement of electricity during the year y in tons of CO2

PEy		project emissions during the year y in tons of CO2

Ly		leakage caused due to project activity in tons of CO2

ERelectricity,y

In terms of emission reduction due to electricity (ERelectricity,y), the only source in the project activity is through the generation of electricity.  The calculation of these emission reductions is as follows:

ERelectricity=EGy.EFy

Where:

EGy		quantity of electricity exported to the grid (MWh)

EFy	grid based emission factor, determined through the combined margin approach as set out in ACM0002 tCO2e/MWh

The calculation of EFy is carried out through the application of relevant sections of methodology ACM0002 version 6.  The sources of data for the combined margin, EFy, are presented in the monitoring plan and this variable will be calculated ex post from the average of the Simple Operating Margin (OM) and the Build Margin (BM).  The application of the methodology does require the use of default

values for the weightings applied to the Simple OM and BM and we have applied the standard weights of 50:50.

The use of the Simple OM requires us to show that the proportion of low-cost/must run resources are less than 50% of total generation in the average of the last 5 years of data.  Low cost/must-run resources typically include hydro, geothermal, wind/ low cost biomass nuclear and solar generation.  Low-cost/must run resources identified are identified as hydro and nuclear (the CEA does not provide any generation data from low-cost biomass and wind resources in the Western Region).  The following table demonstrates the low percentage that low-cost/must run sources constitute of total generation and therefore confirms the choice of Simple OM, however this may change over the life of the project.

Units operating in the Western Region

|  | 2002-3 Generation,GWh | 2003-4 Generation,GWh | 2004-5 Generation,Gwh | 2005-6 Generation,Gwh |
| --- | --- | --- | --- | --- |
| Thermal | 116701 | 140,296 | 141,678 | 164959 |
| Nuclear | 8642 | 5,673 | 6,203 | 6081.57 |
| Hydro | 30221 | 9,393 | 8,061 | 15830.95 |
| Total | 155564 | 155,362 | 155,942 | 186871.5 |
| Hydro/nuclear as % of total | 33% | 9.70% | 9.15% | 11.75% |

Source: CEA Generation report, http://www.cea.nic.in/newweb/opt2_mon_gena.htm

The following equations are used for the calculation of the Simple OM:

EFOM,y=i,jFi,j,y.COEFi,jjGENj,y

Where:

Fi,j	amount of fuel i (in mass or volume) consumed by relevant power sources j in year(s) y

j	power sources delivering electricity to the grid, not including low operating cost and must run power plants, and including imports to the grid

COEFi,j	CO2 emission coefficient of fuel i (tCO2/mass or volume unit of the fuel) taking into account the carbon content of the fuels used by relevant power sources j and the percent of oxidation of the fule in year(s) y

GENj,y	electricity (MWh) delivered to the grid by source j

And

COEFi=NCVi.EFCO2,i.OXIDi

Where:

NCVi	net calorific value (energy content) per mass or volume unit of fuel i

EFCO2,i	CO2 emission factor per unit of energy of the fuel i

OXIDi	oxidation factor of the fuel

The following equations are used for the calculation of the Build Margin:

EFBM,y=i,mFi,m,y.COEFi,mmGENm,y

Where:

Fi,m,j, COEFi,m, GENm,y are analogous to the variables described for the Simple OM.

The combined margin is calculated from the following equation:

EFy=wOM.EFOM,y+wBM.EFBM,y

Where the weights wOM and wBM are by default 50%.

Project Emissions

The factory plans to purchase biomass residues from outside and therefore emissions related to transport are considered as follows:

PETy=Ny⋅AVDy⋅EFkm,CO2,y

where:

PETy	CO2 emissions during the year y due to transport of the biomass residues to the project plant (tCO2/yr)

Ny	Number of truck trips during the year y

AVDy 	Average round trip distance (from and to) between the rice husk fuel supply sites and the site of the project plant during the year y (km),

EFkm,CO2,y	Avergae CO2 emissions factor for the trucks measured during the year y (tCO2/km)

There is no provision to fire fossil fuels in the boilers and therefore emissions from the use of fossil fuels are not accounted.

Therefore,  PEy = PETy

Leakage

As the energy generating equipments are not transferred from another activity leakage is assumed to be zero.

Therefore, the emission reduction equation reduces to:

ERy=ERelectricity,y−PEy

B.6.2. Data and parameters that are available at validation:

(Copy this table for each data and parameter)

| Data / Parameter: | EFkm,CO2 |
| --- | --- |
| Data unit: | tCO2/km |
| Description: | Emissions factor for transport of biomass |
| Source of data used: | 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Chapter 3: Mobile combustion, Section 3.2.1.2 |
| Value applied: | 0.001108 |
| Justification of the choice of data or description of measurement methods and procedures actually applied : | The data from the plausible options in Table 1-32 has been chosen most conservatively, i.e. we have chosen data that results in the highest emission factor which will result in the largest transport emissions. |
| Any comment: |  |

B.6.3 Ex-ante calculation of emission reductions:

>>

From section B.6.1 the emission reductions are given as:

ERy=ERelectricity,y−PEy

ERelectricity,y=EGy.EFy

Where:

EGy		quantity of electricity exported to the grid annually (MWh)

EFy	grid based emission factor, determined through the combined margin approach as set out in ACM0002 tCO2e/MWh

EGy = 48,774 MWh

EFy has been set at 0.81 tCO2e/MWh

The emissions due to transport of the biomass residue to the project plant are estimated as:

PETy=Ny⋅AVDy⋅EFKm,CO2

Where

PETy 		CO2 emissions during the year y due to transport of the rice husk to the project plant (tCO2/yr)

Ny 	Number of truck trips during the period y.

AVDy 	Average round trip distance (from and to) between the rice husk fuel supply sites and the site of the project plant during the year y (km),

EFkm, CO2 Average CO2 emission factor for the trucks measured during the year y (tCO2/km)

Ny = 2,752

AVDy = 100 km

EFkm,CO2 = 0.00110799 tCO2/km

This gives emissions from transportation of biomass as:

PETy = 305 tCO2e

PEy = PETy = 305 tCO2e

Therefore,  ERy= ERelectricity,y – PEy

= 39,507 - 305

= 39,202 tCO2

B.6.4 Summary of the ex-ante estimation of emission reductions:

>>

| Year | Estimation of project activity emissions (tonnes of CO2e) | Estimation of baseline emissions (tonnes of CO2e) | Estimation of leakage (tonnes of CO2e) | Estimation of overall emission reductions (tonnes of CO2e) |
| --- | --- | --- | --- | --- |
| Year 2007-2008 | 305 | 39,507 | 0 | 39,202 |
| Year 2008-2009 | 305 | 39,507 | 0 | 39,202 |
| Year 2009-2010 | 305 | 39,507 | 0 | 39,202 |
| Year 2010-2011 | 305 | 39,507 | 0 | 39,202 |
| Year 2011-2012 | 305 | 39,507 | 0 | 39,202 |
| Year 2012-2013 | 305 | 39,507 | 0 | 39,202 |
| Year 2013-2014 | 305 | 39,507 | 0 | 39,202 |
| Total tonnes of CO2e | 2,134 | 276,549 | 0 | 274,414 |

B.7 Application of a monitoring methodology and description of the monitoring plan:

| B.7.1 Data and parameters monitored: |  |  |
| --- | --- | --- |
| (Copy this table for each data and parameter) |  |  |
| Data / Parameter: | EGy |  |
| Data unit: | MWh |  |
| Description: | MWh exported to the grid |  |
| Source of data: | Plant records maintained by the power plant manager and sales to MSEB |  |
| Value of data | 48,774 MWh |  |
| Description of measurement methods and procedures to be applied: | Data from the turbine generator will be continuously recorded by the turbine operator. This will be collated at the end of each day and reported to the head of power plant. This will form the basis for calculations and will be tallied against the data record by the MSEB which will be taken monthly by the factory and officials from MSEB. In case there is a difference between the factory records and the MSEB record, the MSEB record will prevail. |  |
| QA/QC procedures to be applied: | The invoices generated for the sale of power to the grid will form a QA/QC check. |  |
| Any comment: | Data will be kept for the crediting period and two years thereafter. |  |

| Data / Parameter: | Ny |
| --- | --- |
| Data unit: | Integer |
| Description: | Number of trips undertaken to transport biomass to the project site |
| Source of data to be used: | Transporter receipts |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 2,752 |
| Description of measurement methods and procedures to be applied: | Each truck that enters the factory will be recorded at the weighbridge from which the number of trucks and will be established. The data will be kept for the later of, two years after the end of the crediting period or the last issuance of CERs for the project activity. 100% of the data will be monitored. |
| QA/QC procedures to be applied: | Procedures to cross check this with financial statements may be provided. |
| Any comment: |  |

| Data / Parameter: | AVDy |
| --- | --- |
| Data unit: | Km |
| Description: | Average return distance |
| Source of data to be used: | Transporter receipts |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 100 |
| Description of measurement methods and procedures to be applied: | The average return distance will be recorded for each truck on entry to the factory. The data will be kept for the later of, two years after the end of the crediting period or the last issuance of CERs for the project activity. 100% of the data will be monitored. |
| QA/QC procedures to be applied: | This data may be cross checked with payments for transportation of the material. |
| Any comment: |  |

| Data / Parameter: | EFy |
| --- | --- |
| Data unit: | tCO2/MWh |
| Description: | Emission factor |
| Source of data to be used: | Calculated from the weighted average of the Simple Operating Margin and Build Margin |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | As this will be determined ex-post we have used for the purposes of our calculations an EF value published by the CEA, 0.81 tCO2e/MWh |
| Description of measurement methods and procedures to be applied: | Calculated variable. |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | This variable will be calculated from the EFOM,y and EFBM,y. Whilst the basis of the calculations have used the CEA published CEF we believe there are issues with the determination of this in terms of the transparency of its calculation, therefore calculated factors at the time of verification may differ from this and cause variations in the actual CERs relative to the expected CERs presented in the PDD. Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | EFOM,y |
| --- | --- |
| Data unit: | tCO2/MWh |
| Description: | Simple Operating Margin |
| Source of data to be used: | Calculated variable but may be taken from government or reputed published data for EFs in India if these are available and calculated on the most up to date data. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | This is calculated from the equations provided in ACM0002 |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | Given the data available in India, it is expected that this will be a Simple Operating Margin. If data is not available for the year in which exports have occurred this variable will be calculated from the most recent data available. Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | EFBM,y |
| --- | --- |
| Data unit: | tCO2/MWh |
| Description: | Build Margin |
| Source of data to be used: | Calculated variable but may be taken from government or reputed published data for EFs in India if these are available and calculated on the most up to date data. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | This is calculated from the equations provided in ACM0002 |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | If data is not available for the year in which exports have occurred this variable will be calculated from the most recent data available. Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | Fi,y |
| --- | --- |
| Data unit: | Mass or volume |
| Description: | Amount of each fossil fuel consumed by each power source/plant |
| Source of data to be used: | Central Electricity Authority, most recent General Review or other publication that contains fossil fuel consumption by power plants. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Measured by individual plants and reported to the CEA. |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | Currently the most recent General Review published is for 2006, http://www.cea.nic.in/power_sec_reports/general_review/ index_general_Review.html |

| Data / Parameter: | COEFi |
| --- | --- |
| Data unit: | tCO2/mass or volume unit |
| Description: | Emission factor |
| Source of data to be used: | India’s Initial National Communication to the UNFCCC, http://natcomindia.org/pdfs/chapter2.pdf for the NCV and EF and IPCC data for OXID |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Measured by relevant Indian ministries or agency on their behalf. |
| QA/QC procedures to be applied: | Check against IPCC values |
| Any comment: | Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | GENj/k/n,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Electricity generation of each power source/plant j, k n |
| Source of data to be used: | Central Electricity Authority, most recent data is from the Monthly Generation Report |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Measured by power plants and reported to CEA |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | Currently the most recent generation data is published by the CEA on the following url http://www.cea.nic.in/god/opm/Monthly_Generation_Report/index_ Monthly_Generation_Report.html Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | Plant name |
| --- | --- |
| Data unit: | Text |
| Description: | Identification of power source/plant for the calculation of the OM |
| Source of data to be used: | Central Electricity Authority, monthly generation reports |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Determined from all plants operating on regional grid |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | Currently the most recent generation data is published by the CEA on the following url http://www.cea.nic.in/god/opm/Monthly_Generation_Report/index_ Monthly_Generation_Report.html Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | Plant name |
| --- | --- |
| Data unit: | Text |
| Description: | Identification of power source/plant for the calculation of the BM |
| Source of data to be used: | Central Electricity Authority, state electricity boards and NTPC websites. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Determined from all plants operating on regional grid |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | Data will be held during the crediting period and two years thereafter. |

| Data / Parameter: | GENj/k/ll, y, IMPORTS |
| --- | --- |
| Data unit: | kWh |
| Description: | Electricity imports to the project electricity system |
| Source of data to be used: | Central Electricity Authority |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Reported by Central Electricity Authority |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | http://cea.nic.in/planning/c%20and%20e/Government%20of% 20India%20website.htm Data will be held throughout the crediting period and 2 years thereafter. |

| Data / Parameter: | COEFi,j, y, IMPORTS |
| --- | --- |
| Data unit: | tCO2/mass or volume unit |
| Description: | CO2 emission coefficient of fuels used in connected electricity systems |
| Source of data to be used: | India’s Initial National Communication to the UNFCCC, http://natcomindia.org/pdfs/chapter2.pdf for the NCV and EF and IPCC data for OXID |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | See EFy |
| Description of measurement methods and procedures to be applied: | Measured by relevant Indian ministries or agency on their behalf. |
| QA/QC procedures to be applied: | Left blank on purpose |
| Any comment: | Data will be held throughout the crediting period and 2 years thereafter. |

B.7.2 Description of the monitoring plan:

>>

The CDM data will be collected monthly and be kept in the plant records.  A detailed monitoring and verification report will be produced by the plant and this will form the basis of the roles and responsibilities and collection frequency of the data required to monitor the project activity.

More generally the generation data from the turbine will however be continuously recorded by current transformers and a manual hourly record will be made by the turbine operator.  This data will be collated at the end of each day and reported in the daily operating report to the factory management, the responsibility for which will be with the Power Plant Manager.  This data will form the basis of the ongoing calculation which will then be tallied against the monthly recordings taken by the MSEB and a representative of the factory.

The sugar factory has a Quality Management System certificate ISO:9001-2000 and this will make sure that the factory has well defined procedures for the collection of data in general.

The organization has trained the staff to ensure that the monitoring process is appropriate and effective and the evidence for the same have been provided to the DOE during validation.

The calculation of the ex-post EF will be carried out annually in line with the guidance in the monitoring section.  This will be performed by Agrinergy and all data sources used for the calculation will be kept for the period of the crediting period and two years thereafter.

B.8 Date of completion of the application of the baseline and monitoring methodology and the name of the responsible person(s)/entity(ies)

>>

27/12/2006

Ben Atkinson, Agrinergy Ltd - Project Participant

Ajit Kulkarni, Shree Chhatrapati Shahu Co-operative Sugar Factory – Project Participant

Contact information as listed in Annex 1.

SECTION C. 	Duration of the project activity / crediting period

C.1	Duration of the project activity:

C.1.1.	Starting date of the project activity:

>>

09/01/2006 – Letter of Intent for the purchase of 12.5MW turbine generator.

C.1.2.	Expected operational lifetime of the project activity:

>>

20years 0 months

C.2	Choice of the crediting period and related information:

A seven year renewable crediting period has been chosen.

C.2.1.	Renewable crediting period

C.2.1.1.	 	Starting date of the first crediting period:

>>

15/08/2007 or the date of registration whichever is later.

C.2.1.2.		Length of the first crediting period:

>>

7 years  0 months

C.2.2.	Fixed crediting period:

C.2.2.1.		Starting date:

>>

Not applicable.

C.2.2.2.		Length:

>>

Not applicable.

SECTION D. 	Environmental impacts

>>

D.1.	If required by the host Party, documentation on the analysis of the environmental impacts of the project activity:

>>

In relation to the baseline scenario no negative environmental impacts will arise as a result of the project activity.

The positive environmental impacts arising from the project activity are:

- • A reduction in carbon dioxide emissions from the replacement of fossil fuels which would be generated under the baseline scenario

- • A reduction in the emissions of other harmful gases (NOx and SOx) that arise from the combustion of coal in power generation

The factory will meet all local and national environmental legislation and this will be verified by the continuous monitoring by the Maharashtra Pollution Control Board.  The factory has also conducted an EIA.

A “Consent to Establish” has been obtained by the factory .  The “Consent to Operate” has been provided to Shree Chhatrapati Shahu Co-operative Sugar Factory Ltd in previous years when they operated a bagasse based captive power plant and thus shows that the factory complied with existing environmental legislation.  The consent is given on the basis that the factory operates within the prescribed limits of the Air and Water Act.  In the case of the factory these are air emissions of less than 150 mg/Nm3 for particulate matter and trade effluent not exceeding 30 m3/day and limits on BOD and COD will not exceed 100mg/l and 250mg/l respectively.

The “Consent to Operate” will be obtained annually from the MPCB and this will be provided at the time of each annual verification.

D.2.	If environmental impacts are considered significant by the project participants or the host Party, please provide conclusions and all references to support documentation of an environmental impact assessment undertaken in accordance with the procedures as required by the host Party:

>>

The environmental impacts are not considered significant.

SECTION E. 	Stakeholders’ comments

>>

E.1.	Brief description how comments by local stakeholders have been invited and compiled:

>>

The stakeholder review has been conducted on three levels:

- A local stakeholder review

- A national stakeholder review which will be undertaken through the approval by the Ministry of Environment and Forests and consent to operate from the Maharashtra Pollution Control Board

- An international stakeholder review which will be conducted at the time of validation

The institutions are already in place for the national and international stakeholder review and any comments arising from these processes will be incorporated prior to registration.  The project has received approvals from the Indian designated national authority (the Ministry of Environment and Forests) and the UK designated national authority (DEFRA).

The local stakeholder review has been conducted by the publication of notice on 1st February, 2007 in Indian Express (Pune Edition) and on 30th January, 2007 in Daily Sakal (local marathi newspaper).  A no objection certificate has been obtained from the local municipality.

E.2.	Summary of the comments received:

>>

The comments received mainly revolved around the benefits to the local community and availability of electricity in the local areas.  Further, the stakeholders asked about any adverse effect from the project activity to the local community or to the surrounding environment.

E.3.	Report on how due account was taken of any comments received:

>>

The project proponent answered all the comments in detail and explained the benefits of running a power plant from a renewable source like bagasse as compared to using fossil fuels.  The concept of CDM was also explained and the resulting benefits to the sugar factory and its shareholders.  Further, it was made clear that the project activity does not lead to any impact on environment as it uses bagasse a renewable fuel and the power plant will adhere to the regulations from the pollution control board.  Further, the sugar factory is a cooperative and the benefits from CDM will ultimately reflect in the returns to the farmers who are the shareholders.

Annex 1

# CONTACT INFORMATION ON PARTICIPANTS IN THE PROJECT ACTIVITY

| Organization: | Shree Chhatrapati Shahu Co-operative Sugar Factory Ltd |
| --- | --- |
| Street/P.O.Box: |  |
| Building: | Shrimant Jayashingrao Ghatge Bhavan |
| City: | Kagal, Dist- Kolhapur |
| State/Region: | Maharashtra |
| Postfix/ZIP: | 416 216 |
| Country: | India |
| Telephone: | +91-2325 244211 to 244214 |
| FAX: | +91-2325 244241 |
| E-Mail: | ajitvkulkarni@rediffmail.com |
| URL: |  |
| Represented by: |  |
| Title: | Mr. |
| Salutation: |  |
| Last Name: | Kulkarni |
| Middle Name: | V. |
| First Name: | Ajit |
| Department: |  |
| Mobile: | +91-9422045312 |
| Direct FAX: |  |
| Direct tel: | +91-2325-244211 |
| Personal E-Mail: |  |

| Organization: | Agrinergy Ltd |
| --- | --- |
| Street/P.O.Box: |  |
| Building: | Suite 205, Eagle Tower |
| City: | Cheltenham |
| State/Region: | Montpellier Drive |
| Postfix/ZIP: | GL50 1TA |
| Country: | UK |
| Telephone: | +44 1425 206345 |
| FAX: | +44 1425 206346 |
| E-Mail: |  |
| URL: | www.agrinergy.com |
| Represented by: |  |
| Title: | Director |
| Salutation: | Mr |
| Last Name: | Atkinson |
| Middle Name: |  |
| First Name: | Ben |
| Department: |  |
| Mobile: | +44 7960 970974 |
| Direct FAX: | +44 1425 206346 |
| Direct tel: | +44 1425 206345 |
| Personal E-Mail: | ben.atkinson@agrinergy.com |

Annex 2

# INFORMATION REGARDING PUBLIC FUNDING

The project has not received any public funding.

# Annex 3

BASELINE INFORMATION

This section has been left blank on purpose.

## Annex 4

MONITORING INFORMATION

Please refer to section B.7.1 and B.7.2.

- - - - - -

http://www.mapsofindia.com/lat_long/maharashtra/maharashtra.htm

http://cdm.unfccc.int/methodologies/SSCmethodologies/approved.html

AMS-1.D  - http://cdm.unfccc.int/methodologies/SSCmethodologies/approved.html

This project also provided assistance to the only other registered Indian sugar cooperative project ref: 0313.

http://www.indiatogether.org/2007/apr/agr-sugarcoop.htm

The figure of 20% has been confirmed through discussions with Directors of Indian private sugar factories.  This figure is further demonstrated to be reasonable by the calculation of the weighted average cost of capital of the 4 largest sugar companies in India.  The average of the WACC of these 4 companies (Bajaj Hindusthan, Balrampur Chini, Triveni and Dhampur) is 20.7%.

In the financial analysis presented Rupees lacs refers to Rs 100,000.

EFy will be determined ex-post but for the purposes of the calculation we have taken the CEA published data.  At the time of verification this factor will be re-calculated.