- Annex 6.2 Additional information to be considered as part of the NIR submission (where relevant) or other useful reference information – Greenland/Faroe Islands

6.2.1 CO2 emissions in Greenland and the Faroe Islands

In the Faroe Islands a major work was made in 2002 to produce a revised and more comprehensive greenhouse gas inventory as required by the IPCC guidelines (Lastein et al., 2003). The work comprised emission estimates of CO2, CH4 and N2O for the years 1990-2001.

An update of this work has recently been made (Heilsufrøðiliga Starvsstovan, 2005). The results reported in the latter work are however incomplete as regards the emission sources included, and thus the current 2002 estimate is used also for 2003 and 2004.

The significant increase in CO2 emissions from 1998 to 2001 is mainly due to more fuel use in the fishery, public electricity and manufacturing industry sectors, while the CH4 and N2O emission increases (the Faroe Islands) are due to rising activity in the agricultural sector.

For Greenland the inventory has been expanded to include emissions from agriculture and consumption of F-gases and the pollutants CH4, N2O, HFCs, CO, NMVOC and NOx. However, fossil fuels are still the most important sources of greenhouse gases in this region. Figures for CO2, CH4, and N2O emissions from 1990 to 2004 and for HFCs for 1995-2004 are given in the table below. The inventory is based on information from e.g. KNI Pilersuisoq, Statoil, Nukissiorfiit Årsoversigt 2004, Grønlands Kommando (Greenland Command) and Konsulenttjenesten for Landbrug (Consultancy for Agriculture). The methodology applied is described in Annex 6.2.1.

Table 1 Estimation of greenhouse gas emissions in Greenland and the Faroe Islands 1990-2004.

|  | Greenland |  |  |  | Faroe Islands |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gg CO2 | Mg CH4 | Mg N2O | HFCs Gg CO2-eq. | Gg CO2 | Mg CH4 | Mg N2O |
| 1990 | 625 | 769 | 27,5 |  | 709 | 853 | 73 |
| 1991 | 610 | 775 | 27,2 |  | 682 | 885 | 78 |
| 1992 | 596 | 776 | 26,8 |  | 650 | 881 | 78 |
| 1993 | 5461 | 777 | 25,6 |  | 536 | 875 | 73 |
| 1994 | 496 | 779 | 24,4 |  | 544 | 906 | 79 |
| 1995 | 535 | 785 | 25,5 | 0,060 | 541 | 909 | 81 |
| 1996 | 598 | 783 | 26,7 | 0,080 | 578 | 904 | 81 |
| 1997 | 618 | 782 | 27,2 | 0,39 | 559 | 935 | 86 |
| 1998 | 579 | 762 | 26,2 | 0,71 | 616 | 908 | 83 |
| 1999 | 594 | 732 | 26,7 | 1,27 | 645 | 920 | 87 |
| 2000 | 667 | 690 | 28,3 | 1,85 | 699 | 959 | 97 |
| 2001 | 617 | 681 | 27,0 | 2,93 | 791 | 973 | 101 |
| 2002 | 579 | 647 | 25,7 | 3,86 | 791 | 973 | 101 |
| 2003 | 637 | 673 | 27,0 | 4,70 | 791 | 973 | 101 |
| 2004 | 670 | 678 | 28,4 | 5,35 | 791 | 973 | 101 |

- The CO2 emission for 1993 is intrapolated.

References

Heilsufrøðiliga Starvsstovan 2005: Útlát av veðurlagsgassi í Føroyum – Uppgerð dagført fram til 2003. Heilsufrøðiliga Starvsstovan: 20 pp. Available at: http://www.hfs.fo/tíðindaskriv/tidindi.asp.

Lastein, L. & Winther, M. 2003: Emission of greenhouse gases and long-range transboundary air pollutants in the Faroe Islands 1990-2001. National Environmental Research Institute. - NERI Technical Report 477 (electronic): 62 pp. Available at: http://www.dmu.dk/1_viden/2_Publikationer/3_fagrapporter/rapporter/FR477.PDF

## 6.2.2 Methodology applied for the GHG inventory for Greenland

The GHG inventory for Greenland includes the following sectors:

- Energy sector

- Industrial processes (consumption of F-gasses)

- Agriculture (sheep)

- Solid waste management (incineration without energy recovery, disposal, and open burning)

The applied methodology do to a large degree follow the methodology applied in the Danish inventory, however, the availability of data – especially site specific data – do allow the same equations to used for all the sectors. The actual methodology is described below for the different sectors. The data handling and calculations were performed by use of the IPCC excel tool (version 1.1). The excel tool were modified/extended to cover all relevant processes and calculations were corrected where it was found necessary.

- Energy sector

The inventory covering the energy sector has been performed according to the IPCC tier 1 methodology. The CO2 emission has been calculated by using the methodology included in the IPCC software. This methodology implies use of C content per fuel type (default) and fraction of carbon oxidised (default); see the equation below.

ECO2 = Σ Acta × EFC, a × Ox × 4412

where:

Actfuel =	activity; consumption of fuel a

EFC, fuel =	C emission factor for fuel a

Ox =		oxidation factor

The emissions of CH4, N2O, NOx, CO, and NMVOC have been calculated at sector/fuel level by using IPCC default emission factors combined with measured/Danish EF for waste incineration (with energy recovery). The equation applied for each pollutant is:

E = Σ (EFab × Actab)

where:

EF			emission factor

Act		activity; fuel input

a			fuel type

b			sector activity

- Industrial processes

The inventory covering industrial gasses has been performed according to tier 2 with estimates of the actual emissions. Information on emission of industrial gasses is only available from 1995 onwards.

- Agriculture

Agriculture is sparce in Greenland due to climatic conditions, however sheeps are considered to contribute to emission of greenhouse gasses. Enteric fermentation and manure management is assumed to contribute to emission of CH4 and nitrogen excretion is assumed to contribute to emission of N2O.

The equations used are presented below.

ECH4 = Nsheep × (EFent.ferm. + EFm.m.)

EN2O = Nsheep × Nex. × EFN2O × 4428

The applied emission factors are presented in Table 1.

| Table 1 Applied emission factors1 for agriculture. |  |  |  |
| --- | --- | --- | --- |
|  | Enteric fermentation kg CH4/head/year | Manure management kg CH4/head/year | Nitrogen excretion kg N/head/year |
| Sheep | 17,17 | 0,32 | 16,87 |
| - The emission factors are adopted from the Danish inventory (Illerup et al., 2006). |  |  |  |

The IPCC default for EFN2O: 0,02 kg N2O-N/kg N has been chosen.

- Solid waste management

The solid waste management in Greenland can be divided in the following processes:

- Waste incineration with energy recovery

- Waste incineration without energy recovery

- Managed waste disposal combined with open burning

- Un-managed waste disposal combined with open burning

Information on amount of waste produced per year, amount of waste treated in the different processes, distribution between household and commercial waste, composition of the household waste and commercial waste, respectively, were provided by Ministry of Environment and Nature, Nuuk, Greenland; see Table 2. The distribution of waste between different treatment options after correction for open burning is presented in Table 3. The calculations are based on the information given in Table 2.

| Table 2 Composition of municipal waste before and after open burning. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fraction (%) | Household waste2 | Commercial waste2 | Weighted | After open burning | Weighted (after open burning) | DOC3 | DOC weighted (after open burning) |
|  | % | % | % |  | % |  |  |
| Paper/cardboard, dry | 81 | 20 | 11,8 | 2,37 | 7,66 | 0,4 | 3,06 |
| Paper/cardboard, wet | 101 | 7 | 9,04 | 1,81 | 5,85 | 0,2 | 1,17 |
| Plastics | 71 | 9 | 7,64 | 1,53 | 4,94 | 0 | 0 |
| Organic waste | 441 | 34 | 40,8 | 8,16 | 26,4 | 0,2 | 5,28 |
| Other combustible | 17,51 | 16 | 17,0 | 3,40 | 11,0 | 0,2 | 2,20 |
| Glass | 7,51 | 31 | 6,06 | 6,06 | 19,6 | 0 | 0 |
| Metal | 3,51 | 31 | 3,34 | 3,34 | 10,8 | 0 | 0 |
| Other, non combustible | 11 | 5 | 2,28 | 2,28 | 7,37 | 0 | 0 |
| Hazardous waste | 1,51 | 31 | 1,98 | 1,98 | 6,40 | 0 | 0 |
| Total | 100 | 100 | 100 | 30,9 | 100 |  | 0,12 |
| % | 0,684 | 0,324 |  | 0,85 |  |  |  |
| - Measured values. - Source: Ministry of Environment and Nature, Nuuk, Greenland. - Source: Illerup et al. (2006). - Distribution of household and commercial waste. - Share of combustible waste burned at waste disposal sites. |  |  |  |  |  |  |  |

| Table 3 Waste management in Greenland. Waste for disposal is corrected for open burning. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 |
| 6A1 Managed waste disposal corrected for open burning | 5,61 | 5,70 | 5,74 | 5,78 | 5,85 | 5,95 | 5,93 | 5,92 | 5,66 | 5,19 | 4,35 | 4,38 | 4,16 | 4,45 | 4,19 |
| 6A2 Unmanaged waste disposal corrected for open burning | 1,34 | 1,37 | 1,36 | 1,36 | 1,35 | 1,29 | 1,21 | 1,16 | 1,06 | 1,00 | 0,91 | 0,87 | 0,85 | 0,84 | 0,83 |
| Waste incineration, energy recovery | 6,90 | 6,99 | 7,05 | 7,09 | 7,34 | 7,55 | 7,73 | 7,82 | 7,97 | 9,78 | 12,9 | 13,2 | 14,5 | 14,0 | 15,3 |
| Waste incineration | 0,00 | 0,00 | 0,00 | 0,00 | 0,06 | 0,22 | 0,79 | 1,24 | 2,67 | 2,92 | 3,14 | 3,31 | 3,40 | 3,42 | 3,45 |
| Open burning | 15,5 | 15,8 | 15,9 | 16,0 | 16,1 | 16,2 | 15,9 | 15,8 | 15,0 | 13,8 | 11,7 | 11,7 | 11,2 | 11,8 | 11,2 |
| Total | 29,38 | 29,83 | 30,02 | 30,19 | 30,68 | 31,21 | 31,60 | 31,98 | 32,37 | 32,71 | 33,09 | 33,52 | 34,07 | 34,53 | 35,01 |

The calculation of anaerob degradation at the waste disposal sites is done by use of the tier 1 methodology i.e. by using the following equation:

ECH4 = MSW × MCF × DOC × F × 1612 × (1 − OX)

where:

MSW =	amount of waste disposed of at managed/un-managed disposal sites

MCF =	methane correction factor

DOC =	degradable organic carbon

F =		fraction of methane in landfill gas

OX =		oxidation factor

The emission factors aaplied in the calculation of emissions from incineration of waste with and without energy recovery and open burning are based on measured emissions combined with IPCC default emission factors and Danish emission factors; see Table 4.

| Table 4 Emission factors for incineration of waste. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO2-fossil | CO2-biogenic | CH4 | N2O | NOx | CO | NMVOC | SO2 |
|  | Kg | kg | g | g | g | g | g | g |
| EF DK1 |  |  |  |  |  |  |  |  |
| /GJ | 17,6 | 94,5 | 6 | 4 | 164 | 10 | 9 | 67 |
| /t wet | 185 | 992 | 63,0 | 42,0 | 1 722 | 105 | 94,5 | 704 |
| IPCC2006 |  |  |  |  |  |  |  |  |
| Continuous/semi-cont. |  |  |  | 50,0 |  |  |  |  |
| Batch, stoker |  |  | 60,0 | 60,0 |  |  |  |  |
| Open burning /t dry |  |  |  | 150 |  |  |  |  |
| Open burning /t wet |  |  |  | 106 |  |  |  |  |
| Measured, incineration2 |  |  |  |  |  |  |  |  |
| Medium plants /t wet |  |  |  |  | 3 210 | 224 |  | 693 |
| Small plants /t wet |  |  |  |  | 3 074 | 31 880 |  |  |
| Applied EF |  |  |  |  |  |  |  |  |
| Waste incineration /t wet | 185 | 992 | 60,0 | 50,0 | 3 210 | 224 |  | 693 |
| Waste incineration /GJ | 17,6 | 94,5 | 5,7 | 4,8 | 305,7 | 21,4 | 9,00 | 66 |
| Waste incineration, Village plant /t wet | 185 | 992 | 60,0 | 60,0 | 3 074 | 31 880 | 94,5 | 693 |
| Open burning /t wet | 185 | 992 | 60,0 | 213 | 3 074 | 31 880 | 94,5 | 693 |
| - Source: Illerup et al. (2006). - Source: Ministry of Environment and Nature, Nuuk, Greenland. |  |  |  |  |  |  |  |  |

- Summary

Time series for the greenhouse gasses – see Table 5 – and for the different sectors – see Table 6 – as well as summary tables for the years 1990, 1995, and 2004 are presented below. CO2 is accounting for more than 95% of the emission of greenhouse gasses and the emission of greenhouse gasses is mainly related to the energy sector.

| Table 5 Time series for emission of greenhouse gasses from 1990-2004. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 |
| CO2 | Gg | 630 | 615 | 601 | 551 | 500 | 540 | 603 | 623 | 584 | 599 | 672 | 622 | 583 | 642 | 670 |
| CH4 | Gg CO2-eq. | 16,2 | 16,3 | 16,3 | 16,3 | 16,4 | 16,5 | 16,4 | 16,4 | 16,0 | 15,4 | 14,5 | 14,3 | 13,6 | 14,1 | 14,2 |
| N2O | Gg CO2-eq. | 8,52 | 8,42 | 8,32 | 7,94 | 7,57 | 7,89 | 8,27 | 8,42 | 8,12 | 8,29 | 8,77 | 8,37 | 7,96 | 8,38 | 8,80 |
| HFC | Gg CO2-eq. |  |  |  |  |  | 0,025 | 0,077 | 0,39 | 0,71 | 1,26 | 1,85 | 2,93 | 3,85 | 4,69 | 5,35 |
| SF6 | Gg CO2-eq. |  |  |  |  |  | 0,0359 | 0,0034 | 0,0034 | 0,0033 | 0,0033 | 0,0033 | 0,0032 | 0,0032 | 0,0032 | 0,0031 |
| Total | Gg CO2-eq. | 655 | 640 | 626 | 575 | 525 | 565 | 628 | 649 | 609 | 625 | 697 | 648 | 609 | 670 | 698 |

| Table 6 Time series for emission of greenhouse gasses from 1990-2004 within different sectors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 |
| Energy | Gg CO2-eq. | 633 | 618 | 603 | 552 | 502 | 541 | 605 | 625 | 585 | 601 | 675 | 625 | 586 | 645 | 673 |
| Industry | Gg CO2-eq. |  |  |  |  |  | 0,06 | 0,08 | 0,39 | 0,71 | 1,27 | 1,85 | 2,93 | 3,86 | 4,70 | 5,35 |
| Agriculture | Gg CO2-eq. | 10,6 | 10,6 | 10,6 | 10,6 | 10,6 | 10,6 | 10,6 | 10,6 | 10,6 | 10,6 | 10,9 | 10,6 | 10,1 | 10,3 | 10,9 |
| Waste management | Gg CO2-eq. | 11,5 | 11,7 | 11,8 | 11,8 | 11,9 | 12,1 | 12,1 | 12,1 | 11,8 | 10,9 | 9,39 | 9,44 | 9,05 | 9,53 | 9,09 |
| Total | Gg CO2-eq. | 655 | 640 | 626 | 575 | 525 | 565 | 628 | 649 | 609 | 625 | 697 | 648 | 609 | 670 | 698 |

1990

1995

2004

- References

Illerup, J.B., Lyck, E., Nielsen, O.-K., Mikkelsen, M.H., Hoffmann, L., Gyldenkærne, S., Nielsen, M., Sørensen, P.B., Fauser, P., Thomsen, M. & Winther, M. (2006). Denmark’s National Inventory Report 2006. Submitted under the United Nations Framework Convention on Climate Change, 1990-2004. NERI Technical Report No. 589.